Series

OAU	Agricultural Wash Down Unit Heater

Suitable for use in Category 1 and 2 areas per Canadian Electrical Code.

Features

Certification	- 	CSA C22.2 46, UL2021.
-	Corrosion requirements CSA C22.2 94.
-	Hose down requirements CSA C22.2 94 Type 4X.

Color	-	Standard: white.
Finish	-	Standard: epoxy/polyester powder paint making it more resistant to corrosion.
Voltage	-	240/208V,1-phase.	
Construction	-	Robust, 18-gauge satin coat steel construction.
	-	Adjustable louvers to direct air flow.
	-	High-limit temperature control with automatic reset.

-	CSA C22.2 94 4X non-metallic electrical enclosure.
Fan	-	Totally enclosed air over and factory-lubricated ball bearing motor.
	-	Motor mounted in cold compartment.
	-	Thermally-protected motor.
	-  Fan delay at start and shutdown.		
Heating element	-	Durable low-density steel tubular heating elements with fins; painted in black

for a better corrosion resistance..
Control	-	40A disconnect switch built-in.
	-	240/208V control circuit built-in.
	- 	Fan switch control built-in.
	-	Built-in pilot light indicating when high-limit temperature control has been activated.
	-	High-quality built-in thermostat with stainless steel bulb and capillary, offering great accuracy.
Installation	-	Horizontal wall or ceiling mounting using one of two supplied brackets  
		which allow 360° rotation.
	-	Sealed electrical enclosure located at bottom of unit for ease of installation and servicing.
	-	Maximum recommended height 12 ft. (3.7 m).
	-	Minimum height of 8 ft. (2.4 m)
	-	30 °C (85 °F) maximum operating ambient temperature.
Warranty	-	1-year warranty against defects.
	-	3-year on the heating elements.
Application 	-	Agricultural installations (dairy farm, stables, poultry farm, barn, pig farm, kennel, etc.).

Model

| Product # | Qty | Volts | Phase | Watts (BTU/hr) | cfm | Temperature Rise |  | Motor |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | oC | oF | HP | rpm | lb | kg |
| OAU05000BLLT-DIS40-IV-T1 |  | 240/208 | 1 | 5000/3750 (17060/12795) | 300 | 30 | 53 | 1/4 | 1050 | 52.8 | 23.9 |

1 Unit with automatic reset high-limit temperature control.

Standard color is white

### Other

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

# Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |