[image: image1.jpg]A\




[image: image4.jpg]@DUELLET



[image: image4.jpg]

Approval Drawings

Series

OCA
Cabinet Unit Heater
[image: image5.jpg]VERSION AVAILABLE




[image: image6.jpg]


 

Features

Color
-
Standard: white, almond. 


-
Optional (upcharge applicable): metallic silver, bronze, metallic charcoal, aluminum, 
semi-gloss black, sand.


-
Custom colors available upon request.

Finish
-
Standard: epoxy/polyester powder paint.

Voltage
-
208V, 240V, 277V, 347V, 480V, 600V, 1 or 3-phase.

Construction
-
18-gauge steel.

-
Insulated casing for noise reduction.

-
Extruded aluminum bar grilles prevent insertion of foreign objects.


-
Field adaptable air inlet and outlet.


-
High-limit temperature control with automatic reset.


-
240V control circuit included.


-
Optional fresh air inlet damper.


-
Optional air ﬁlters.

Fan
-
Direct drive, variable speed squirrel-cage fan.


-
Fan delay.


-
240/208V factory-lubricated motor (1/15 HP).


-
2 speed-motor available (1600/1500 rpm).
Heating element
-
Durable tubular heating element with fins.

Length
-
Four cabinet lengths are available: 28 in. (711 mm), 48 in. (1219 mm), 



66 in. (1676 mm), 80 in. (2032 mm).

Control
-
Wide range of control available.

Ouellet highly recommends use of a wall thermostat for greater comfort.

Installation
-
Multi-position mounting (surface, semi-recessed, recessed).


-
Maximum recommended installation height: 10 ft. (3 m).


-
Recess trim kit available for recessed or semi-recessed units.


-
Sub-base and special back plate available.
Warranty
-
5-year warranty against defects.

Application
-
Hallway, entry way, exhibition center.

[image: image2.jpg]Installation
with Sub-Base

7

51 mm
D'y

254 mm
10"

Installation with Recess Trim Kit

51 mm
o

Recessed

51 mm
o

Semi-Recessed

,

Surface
Installation

686

27"

mm

Wall
Opening

KOE ©44-@35 mm
KOE @1 3/4"-@1 3/8"

Cabinet length +

254 mm (1")

KOE @28-@22 mm

— KOE @1 1/8"-@7/8"

m
6"

L39 mm

112"

Front View

KOE ©6.4 mm
51 if KOE @1/4 l
o
‘ /* 254 mm
o‘o' 10"
* A_—J L 64 mm ’
[ — 21/2"
157 mmi. 187 mm
63/16" 7 3/8"

Bottom View




[image: image3.jpg]KOE ©44-035 mm
KOE @1 3/4"-21 3/8" KOE @28-622 mm

— KOE®1 18078’

KOE @6.4 mm
KOE @1/4"
51 mm
o
686 mm Wall *
27 Opening Bl
1 -
: 2172
Length + 157 mm |
—39mm T 254mm() - 6ang — = 1% mm
o 738
FrontView Bottom View
Installation with Sub-Base Installation
7 7 7
51 mm 51 mm 51 mm
oF JF oF

T

Recessed Semi-Recessed Surface
Edi Installation
10"
Wall Mouting Ceiling
Top Bottom Mounting
L vt
Semi-Recessed Recessed
D Type Mounting Only
Wall
- -
- -
Semi-Recessed Recessed
D Type Mounting Only





Models

	Watts
	Product #
	BTU/hr
	cfm
	Length
	Weight

	Hi
	Lo
	208V
(3 ph)
	Qty
	240V
(1 ph)
	Qty
	240V
(3 ph)
	Qty
	277V
(1 ph)
	Qty
	480V
(3 ph)
	Qty
	Hi
	Lo
	Hi
	Lo
	in.
	mm
	lb
	kg

	2000
	1333
	OCAU020381
	
	OCAU02000
	
	OCAU02030
	
	OCAU02077
	
	OCAU020341
	
	6830
	4552
	250
	200
	28
	711
	115
	52.3

	3000
	1000
	OCAU030381
	
	OCAU03000
	
	OCAU03030
	
	OCAU03077
	
	OCAU030341
	
	10246
	3415
	250
	200
	28
	711
	115
	52.3

	4000
	2667
	OCAU040381
	
	OCAU04000
	
	OCAU04030
	
	OCAU04077
	
	OCAU040341
	
	13661
	9108
	250
	200
	28
	711
	115
	52.3

	5000
	3333
	OCAU050381
	
	OCAU05000
	
	OCAU05030
	
	OCAU05077
	
	OCAU050341
	
	17076
	11383
	250
	200
	28
	711
	115
	52.3

	6000
	4000
	OCAU060381
	
	OCAU06000
	
	OCAU06030
	
	OCAU06077
	
	OCAU060341
	
	20491
	13661
	250
	200
	28
	711
	115
	52.3

	8000
	4000
	OCAU080381
	
	OCAU08000
	
	OCAU08030
	
	OCAU08077
	
	OCAU080341
	
	27321
	13661
	500
	400
	48
	1219
	146
	66.4

	10000
	5000
	OCAU100381
	
	OCAU10000
	
	OCAU10030
	
	OCAU10077
	
	OCAU100341
	
	34152
	17076
	500
	400
	48
	1219
	146
	66.4

	12000
	6000
	OCAU120381
	
	OCAU12000
	
	OCAU12030
	
	OCAU12077
	
	OCAU120341
	
	40982
	20491
	500
	400
	48
	1219
	146
	66.4

	15000
	10000
	OCAU150381
	
	OCAU15000
	
	OCAU15030
	
	-
	-
	OCAU150341
	
	51228
	34152
	750
	600
	66
	1676
	210
	95.5

	18000
	12000
	OCAU180381
	
	OCAU18000
	
	OCAU18030
	
	-
	-
	OCAU180341
	
	61473
	40982
	750
	600
	66
	1676
	210
	95.5

	20000
	10000
	OCAU200381
	
	OCAU20000
	
	OCAU20030
	
	-
	-
	OCAU200341
	
	68304
	34152
	1000
	800
	80
	2032
	240
	109.1

	24000
	12000
	OCAU240381
	
	OCAU24000
	
	OCAU24030
	
	-
	-
	OCAU240341
	
	81964
	40982
	1000
	800
	80
	2032
	240
	109.1


1 1-phase available upon request (change the "3" for an "0" to the product number).

Add "BL" for white, "AM" for almond. Upcharge available for optional colors shown in the color chart section.

Options
	Product # 

Kit
	Product #

Factory installed*
	Description
	Qty

	-
	120
	120V control circuit 
	

	-
	3P
	Triple-pole contactor
	

	-
	AF1
	Fresh air inlet
	

	-
	AFM 1
	Power-operated fresh air inlet
	

	-
	DIS40 4
	Disconnect switch, 40A
	

	-
	DIS80 4
	Disconnect switch, 80A
	

	-
	DIS100 4
	Disconnect switch, 100A
	

	-
	FUS
	Circuit protection fuse
	

	-
	HL2
	Hi/Lo power switch
	

	KIT-OCA-IV
	IV
	On/Auto fan switch
	

	-
	MR
	High-limit temperature control with manual reset
	

	KIT-OCA-OO
	OO
	On/Standby heating switch
	

	KIT-OCA-R
	R
	24V control relay, without transformer
	

	-
	RHL3
	24V Hi/Lo power relay, with transformer
	

	-
	ROO3
	24V On/Standby heating relay, with transformer
	

	KIT-OCA-RT
	RT
	24V control relay, with transformer
	

	-
	RV3
	24V fan relay, with transformer
	

	KIT-OCA-T-AV
	TAV
	Single-pole thermostat, tamperproof
	

	-
	T2S
	2-stage thermostat, tamperproof
	

	Recess trim kit, 2 in. (51 mm)

	KIT-OCA-CR-28
	-
	28 in. (711 mm) Models
	

	KIT-OCA-CR-48
	-
	48 in. (1219 mm) Models
	

	KIT-OCA-CR-66
	-
	66 in. (1676 mm) Models
	

	KIT-OCA-CR-80
	-
	80 in. (2032 mm) Models
	

	Special back plate

	KIT-OCA-DU-28
	-
	28 in. (711 mm) Models
	

	KIT-OCA-DU-48
	-
	48 in. (1219 mm) Models
	

	KIT-OCA-DU-66
	-
	66 in. (1676 mm) Models
	

	KIT-OCA-DU-80
	-
	80 in. (2032 mm) Models
	

	Air filter, wall installation only

	KIT-OCA-FI-28
	FI
	28 in. (711 mm) Models
	

	KIT-OCA-FI-48
	FI
	48 in. (1219 mm) Models
	

	KIT-OCA-FI-66
	FI
	66 in. (1676 mm) Models
	

	KIT-OCA-FI-80
	FI
	80 in. (2032 mm) Models
	

	Washable air filter, wall installation only

	KIT-OCA-FIW-28
	FIW
	28 in. (711 mm) Models
	

	KIT-OCA-FIW-48
	FIW
	48 in. (1219 mm) Models
	

	KIT-OCA-FIW-66
	FIW
	66 in. (1676 mm) Models
	

	KIT-OCA-FIW-80
	FIW
	80 in. (2032 mm) Models
	

	Sub-base, 2 in. (51 mm)

	KIT-OCA-SO-28
	-
	28 in. (711 mm) Models
	

	KIT-OCA-SO-48
	-
	48 in. (1219 mm) Models
	

	KIT-OCA-SO-66
	-
	66 in. (1676 mm) Models
	

	KIT-OCA-SO-80
	-
	80 in. (2032 mm) Models
	


* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover. 

1 Do not install a built-in thermostat with these options.

2 OCAU06006, is not available with optional HL.

3 When one of these options is ordered, the heater control will automatically be shipped at 24V.

4 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect. 

Others

	Type
	Quantity
	Model et description

Complementary information
	Volts
	Phase
	Watts
	Length

mm/in.
	Information

Color/installation

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	


Specifications may change without prior notice

	Project:
	Approval

Information

	Consultant Engineer:
	

	Eectrician Contractor:
	

	Distributor:
	

	Date:
	

	Contact in Ouellet:
	

	Comments:


180, 3e Avenue, L’Islet (Québec) G0R 2C0 CANADA • Phone: 1 800 463-7043 • Fax: 1 800 662-7801 • info@ouellet.com • www.ouellet.com

