[image: image1.png]120V Model 240/208V Model
L L ° , L
1
Side View Front View Side View Front View



[image: image2.png]


[image: image3.emf]









Approval Drawings

Series

[image: image4.png]


OCF
Patio Infrared Heater
[image: image5.jpg]Grounded 3-pronged outlet (120V model)

NEMA 5-15R
15A, 120VAC







Features

Finish
-
Black anodized aluminum or stainless steel.
Voltage
-
120V, 240/208V, 60 Hz, 1-phase.
Construction
-
Weatherproof IP65 certified for outdoor use. 

-
2 mm wall thickness aluminum extrusion casing.


-
High quality aluminum optical reflector.

-
Safety mechanism that automatically triggers the heater to go into Standby mode 



after 8 hours of operation.

-
High quality power cord:


   120V model: 7.5 ft. (2.25 m) with NEMA 5-15P plug. 



 240/208V models: 6 ft. (1.8 m) hard wire.
Heating element
-
Woven carbon filament infrared lamp inside a 15 mm Quartz tube.

-
Element service life ± 8000h.
Wavelength
-
Mid-wave (2 µm).
Control
-
1500W and 3000W models: On/Off integrated regulator and a remote control 


with 6 settings.
· 4000W and 6000W models: Two heating level, 50% and 100%. Heater can be

wired to 
bypass the remote if requires.
Certification
-
Comply with the standard UL2021:2015 Ed.4 and C22.2 No.46: 2013 Ed.9.
Installation
-
Horizontal installation only.

-
Pivoting wall mounting brackets included (120V and 240/208V models).


-  Mounting brackets for ceiling suspension included (240/208V models).

-
Installation minimum clearances: 


- 1500W model: 6.7 ft. (2.0 m) from floor.



- 300W, 4000W and 6000W models: 8 ft. (2.4 m) from floor.


- 
All models: 12 in. (30 cm) from ceiling, 18 in. (45 cm) between the end of the unit    
  to the wall and 24 in. (61 cm) between the longer side of the unit to the wall.


-
120V Model: Wall mounted only.




Must be plugged into a grounded 3-pronged outlet, 15A, 120VAC. 

-
240/208V models: Wall or ceiling mounted.





Hard-wired into an electrical junction box (not included). 
Warranty
-
1-year warranty against defects.

-
90-day warranty on the element (for industrial applications only). 
Application
-
Resdiential: Solarium, pool & spa, balcony, patio.

· Commercial: Workshop, garage, restaurant patio or bar, hotel entranceway, churches.

[image: image6.jpg]@DUELLET




Models
	Product #1
	Volts
	Watts
	Grounded 3-pronged

outlet
	Dimensions
	Weight
	Qty

	
	
	
	
	L
	H
	D
	
	

	
	
	
	
	in.
	mm
	in.
	mm
	in.
	mm
	lb
	kg
	

	CF15120B
	120
	1500
	15A, 120VAC
	27
	685.8
	6 1/2
	165
	6 5/16
	161
	10
	4.5
	

	CF30240SS2
	240/208
	3000/2250
	-
	38
	965.2
	6 1/2
	165
	6 5/16
	161
	14.3
	6.4
	

	CF40240SS2
	240/208
	4000/3000
	-
	42
	1066.8
	9
	228.6
	9
	228.6
	40
	18.2
	

	CF60240SS2
	240/208
	6000/4500
	-
	42
	1066.8
	9
	228.6
	9
	228.6
	40
	18.2
	


1Mounting brackets included. Finish: “B” for black anodized aluminum, “SS” for stainless steel.
2Indoor residential installation forbidden.
Others

	Type
	Quantity
	Model and description

Complementary informations
	Volts
	Phase
	Watts
	Length

mm/in.
	Informations

Color/installation

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	


Specifications may change without prior notice

	Project:
	Approval

Information

	Consultant Engineer:
	

	Electrician Contractor:
	

	Distributor:
	

	Date:
	

	Contact in Ouellet:
	

	Comments:


180, 3e Avenue, L’Islet (Québec) G0R 2C0 CANADA • Phone: 1 800 463-7043 • Fax: 1 800 662-7801 • info@ouellet.com • www.ouellet.com

