[image: image1.jpg]




Approval Drawings

Series

ODB
Aluminum Draft Barrier

[image: image5.jpg]@DUELLET



    [image: image2.emf]







 [image: image3.jpg]




Features

Color
-
Standard: white, almond. 


-
Optional (upcharge applicable): metallic silver, bronze, metallic charcoal, 



aluminum, semi-gloss black, sand.  


-
Custom colors available upon request.

Finish
-
Standard: epoxy/polyester powder paint.


-
Optional: anodized aluminum.

Voltage
-
120V, 208V, 240/208V, 277V, 347V, 480V, 600V, 50/60 Hz, 1-phase.
Construction
-
14-gauge (0.0787 in. [2 mm]) aluminum front cover.


-
18-gauge satin coat steel cabinet.


-
Screw assembly for added strength.


-
Full length built-in wireway.


-
Pencil-proof openings less than 1/4 in. (6.4 mm).


-
Front fresh air inlet; top warm air outlet.



- Bottom fresh air inlet available (-BAI).

-
Clean back available (-BD).


-
Linear high-limit temperature control with automatic reset. 
High altitude version available (-HA).


-
End caps included. 

Heating element
-
Stainless steel tubular heating element with aluminum ﬁns.

-
Floating heating element on high-temperature nylon bushings


reducing expansion noises.

Watt density
-
Average standard watt density of 275W/ft. 

-
Average low watt density of 100W/ft., 150W/ft., 200W/ft.

Control
-
Built-in thermostat available with a control knob 



or tamperproof.

· Mechanical, electronic or pneumatic relays available 


(installed on the right end only).

Ouellet highly recommends to use an electronic wall thermostat for greater comfort.
Installation
-
Heater must be installed at least 2 in. (51 mm) from finished floor (for -BAI option).


-
Mounting holes spaced on 4 in. (102 mm) intervals for ease of installation.


-
Wall or floor mounted installation. 


-
Variety of accessories available for wall-to-wall installation.

-
Pedestal kit available.

Warranty
-
10-year warranty against defects.

Application
-
Commercial building, office building, hospital, daycare center, private residence.


[image: image4.emf]








Average Standard Watt Density Models 275W/ft.
	Watts
	Watts

240/208V1
	Product  #
	Length
	Weight

	
	
	120V
	Qty
	208V
	Qty
	240/208V
	Qty
	277V
	Qty
	480V
	Qty
	in.
	mm
	lb
	kg

	500
	500/375
	ODB0502
	
	ODB0508
	
	ODB0500
	
	ODB0577
	
	-
	-
	29 1/16
	738
	8.0
	3.6

	750
	750/563
	ODB0752
	
	ODB0758
	
	ODB0750
	
	ODB0777
	
	ODB0754
	
	39 1/16
	992
	10.4
	4.7

	1000
	1000/750
	ODB1002
	
	ODB1008
	
	ODB1000
	
	ODB1077
	
	ODB1004
	
	49 5/16
	1253
	12.8
	5.8

	1250
	1250/938
	ODB1252
	
	ODB1258
	
	ODB1250
	
	ODB1277
	
	ODB1254
	
	58 13/16
	1494
	15.1
	6.8

	1500
	1500/1125
	ODB1502
	
	ODB1508
	
	ODB1500
	
	ODB1577
	
	ODB1504
	
	67 1/2
	1715
	17.2
	7.8

	1750
	1750/1313
	-
	-
	ODB1758
	
	ODB1750
	
	ODB1777
	
	ODB1754
	
	76 7/8
	1952
	19.4
	8.8

	2000
	2000/1500
	-
	-
	ODB2008
	
	ODB2000
	
	ODB2077
	
	ODB2004
	
	85 5/8
	2175
	21.5
	9.7

	2250
	2250/1688
	-
	-
	ODB2258
	
	ODB2250
	
	ODB2277
	
	ODB2254
	
	94 1/2
	2400
	23.6
	10.7

	2500
	2500/1875
	-
	-
	ODB2508
	
	ODB2500
	
	ODB2577
	
	ODB2504
	
	103 5/8
	2632
	25.8
	11.7


Average Low Watt Density Models
	Product  #2
	Length
	Weight
	Average

Density W/ft.
	Watts
	Watts

240/208V1
	120V
	Qty
	208V
	Qty
	240/208V
	Qty
	277V
	Qty
	480V
	Qty

	ODB020X-738
	29 1/16"

(738 mm)
	8.0 lb
(3.6 kg)
	100
	200
	200/150
	(
	
	(
	
	(
	
	-----
	-----
	-----
	-----

	ODB030X-738
	
	
	150
	300
	300/225
	(
	
	(
	
	(
	
	-----
	-----
	-----
	-----

	ODB040X-738
	
	
	200
	400
	400/300
	(
	
	(
	
	(
	
	(
	
	-----
	-----

	ODB030X-992
	39 1/16"

(992 mm)
	10.4 lb
(4.7 kg)
	100
	300
	300/225
	(
	
	(
	
	(
	
	(
	
	-----
	-----

	ODB045X-992
	
	
	150
	450
	450/338
	(
	
	(
	
	(
	
	(
	
	-----
	-----

	ODB060X-992
	
	
	200
	600
	600/450
	(
	
	(
	
	(
	
	(
	
	-----
	-----

	ODB040X-1253
	49 5/16"

(1253 mm)
	12.8 lb
(5.8 kg)
	100
	400
	400/300
	(
	
	(
	
	(
	
	(
	
	-----
	-----

	ODB060X-1253
	
	
	150
	600
	600/450
	(
	
	(
	
	(
	
	(
	
	-----
	-----

	ODB080X-1253
	
	
	200
	800
	800/600
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB050X-1494
	58 13/16"

(1494 mm)


	15.1 lb
(6.8 kg)
	100
	500
	500/375
	(
	
	(
	
	(
	
	(
	
	-----
	-----

	ODB075X-1494
	
	
	150
	750
	750/563
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB100X-1494
	
	
	200
	1000
	1000/750
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB060X-1715
	67 1/2"

(1715 mm)


	17.2 lb
(7.8 kg)
	100
	600
	600/450
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB090X-1715
	
	
	150
	900
	900/675
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB120X-1715
	
	
	200
	1200
	1200/900
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB070X-1952
	76 7/8"

(1952 mm)
	19.6 lb
(8.8 kg)
	100
	700
	700/525
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB105X-1952
	
	
	150
	1050
	1050/788
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB140X-1952
	
	
	200
	1400
	1400/1050
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB080X-2175
	85 5/8"

(2175 mm)
	21.5 lb
(9.7 kg)
	100
	800
	800/600
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB120X-2175
	
	
	150
	1200
	1200/900
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB160X-2175
	
	
	200
	1600
	1600/1200
	-----
	-----
	(
	
	(
	
	(
	
	(
	

	ODB090X-2400
	94 1/2"

(2400 mm)
	23.6 lb
(10.7 kg)
	100
	900
	900/675
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB135X-2400
	
	
	150
	1350
	1350/1013
	-----
	-----
	(
	
	(
	
	(
	
	(
	

	ODB180X-2400
	
	
	200
	1800
	1800/1350
	-----
	-----
	(
	
	(
	
	(
	
	(
	

	ODB100X-2632
	103 5/8"

(2632 mm)
	26.8 lb
(11.7 kg)
	100
	1000
	1000/750
	(
	
	(
	
	(
	
	(
	
	(
	

	ODB150X-2632
	
	
	150
	1500
	1500/1125
	-----
	-----
	(
	
	(
	
	(
	
	(
	

	ODB200X-2632
	
	
	200
	2000
	2000/1500
	-----
	-----
	(
	
	(
	
	(
	
	(
	

	ODB225X-2632
	
	
	200
	2250
	2250/1688
	-----
	-----
	(
	
	(
	
	(
	
	(
	


1 208V = 75% of wattage at 240V.
2 X = 2 (120V), 8 (208V), 0 (240/208V), 77 (277V), 7 (347V), 4 (480V), 6 (600V).
( = Available.

Add "BL" for white, "AM" for almond. Optional colors shown in the color chart section are available upon request.

Other colors, accessories and finishes are available upon request. Please contact your customer service representative.

Other voltages available upon request.

Options
	Product # 

Kit
	Product # 

Factory installed*
	Description
	Qty

	Finish
	
	
	

	-
	BAI
	Bottom fresh air inlet
	

	-
	BD1
	Clean back
	

	KIT-ODB-CE
	-
	Outside corner, 4 in. (102 mm)
	

	KIT-ODB-CI
	-
	Inside corner, 4 in. (102 mm)
	

	KIT-ODB-JT
	-
	Splice plate, 2 in. (51 mm)
	

	KIT-ODB-JT-PE
	-
	Contour splice plate for BD option and/or heater with pedestals, 2 in. (51 mm)
	

	KIT-ODB-PE
	-
	Round pedestal kit (2), 3 in. (76 mm)
	

	KIT-ODB-PEC
	-
	Square-shaped pedestal kit (2), 2 in. (51 mm)
	

	ODB-SV-XX
	-
	Empty cabinet with length of XX mm (minimum: 154 mm, maximum: 2632 mm)
	

	Controls
	
	
	

	KIT-ODB-DIS204
	DIS204
	Disconnect switch, 20A at 277V max.
	

	KIT-ODB-DIS404
	DIS404
	Disconnect switch, 40A at 600V max.
	

	-
	HA
	Linear high-limit temperature control for high altitude (3000 ft. to 8200 ft.)
	

	-
	PM
	Pneumatic relay, 20A at 600V max.
	

	KIT-ODB-R-347
	R
	Single-pole relay, coil 24V, without transformer: 22A at 120V, 208V, 240V - 19A at 277V - 18A at 347V
	

	KIT-ODB-R-600
	R
	Single-pole relay, coil 24V, without transformer, 12.5A at 480V, 10A at 600V
	

	KIT-ODB-RT-120
	RT
	Single-pole relay, coil 24V, with transformer 120V/24V, 22A at 120V 
	

	KIT-ODB-RT-208
	RT
	Single-pole relay, coil 24V, with transformer 208V/24V, 22A at 208V
	

	KIT-ODB-RT-240
	RT
	Single-pole relay, coil 24V, with transformer 240V/24V, 22A at 240V
	

	KIT-ODB-RT-277
	RT
	Single-pole relay, coil 24V, with transformer 277V/24V, 19A at 277V
	

	KIT-ODB-RT-347
	RT
	Single-pole relay, coil 24V, with transformer 347V/24V, 18A at 347V
	

	KIT-ODB-RT-6002
	RT2
	Single-pole relay, coil 24V, with transformer 600V/24V, 10A at 600V (2-wire only)
	

	KIT-ODB-TB6
	TB6
	Single-pole bi-metal thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V
	

	KIT-ODB-TB263
	TB263
	Double-pole bi-metal thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V
	

	KIT-ODB-TB6-AV
	TB6AV
	Single-pole bi-metal tamperproof thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V
	

	KIT-ODB-TB26-AV3
	TB26AV3
	Double-pole bi-metal tamperproof thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V
	

	KIT-ODB-TRIAC-347
	TRIAC
	Single-pole electronic relay, coil 24V. without transformer, 15A at 347V max.
	

	KIT-ODB-TRIAC-347T
	TRIACT
	Single-pole electronic relay, coil 24V, with transformer: 15A at 208V, 240V, 277V, 347V
	

	KIT-ODB-TRIAC-600
	TRIAC
	Single-pole electronic relay, coil 24V. without transformer, 6A at 600V max.
	


* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover. 

1 For anodized aluminum heaters, the clean back option will be fixed with visible rivets.

2 These 2-wire relays with transformer can't supply of 3-wire low voltage thermostats.

3 "Off" position.

4 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect. 
Others

	Type
	Quantity
	Model et description

Complementary informations
	Volts
	Phase
	Watts
	Length

mm/in.
	Informations

Color/installation

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	


Specifications may change without prior notice

	Project:
	Approval

Information

	Consultant Engineer:
	

	Eectrician Contractor:
	

	Distributor:
	

	Date:
	

	Contact in Ouellet:
	

	Comments:


With bottom


fresh air inlet








180, 3e Avenue, L’Islet (Québec) G0R 2C0 CANADA • Phone: 1 800 463-7043 • Fax: 1 800 662-7801 • info@ouellet.com • www.ouellet.com

