[image: image4.jpg]@DUELLET



[image: image4.jpg]

Approval Drawings

Series
OFE-A
Advantage Electric Furnace


[image: image1.png]


    [image: image2.jpg]




Features
Color
-
Standard: grey. 
Voltage
-
240/208V, 1-phase.
Construction
-
Steel casing, all insulated for reduced heat loss and noise.


-
Dual access panels on casing provide easy access to internal parts.
Fan
-
High quality precision balanced blower for quiet operation and optimal performance.

Motor
-
Standard motor (PSC), permanently lubricated.


-
Variable speed electronically commutated motor (ECM), permanently lubricated.


-
Both motors are rubber mounted for maximum vibration and noise reduction.


-
Relay and printed circuit board (PCB) control automatic operation of motor and allow for the addition of air conditioning or heat pump.
Heating element
-
Heavy duty, reliable 4kW and 5kW helical coil nickel-chromium alloy wire provide superior heat transfer with unrestricted air flow.

· Heating elements can be individually replaced.
· High-limit temperature control with automatic reset.
· Heating element sequence is done by a low voltage printed circuit boards with rectifier/timers to prevent power surges.

Control
-
Heavy duty 24VAC transformer provides power to relays and controls.


-
Easy access built-in breakers with manual reset.


-
Thermostat "On" light indicates a call for heat by the thermostat.


-
“Speed Selector” switch: “Low” or “High”.

-
“Fan Selector” switch: “On” or “Off”.


-
Compatible with 1 or 2-stage thermostat with or without external unit.


-
A 120V terminal block also permits installation of accessories such as humidifier and electronic filter (maximum 2A).

Installation
-
Multi-position installation, sides, back, or bottom air returns.

· Approved for zero clearance installation against combustible surfaces.

· Models with Standard Motor (PSC): Installation is forbidden outside Quebec.

Warranty
-
5-year warranty against defects.
Application
-
For cottages, homes or offices, also approved for mobile homes use. May be installed in a basement, closet, crawl space or any unused area or alcove.

[image: image3.png]927 mm
361/2"

Right Side View

Identical on the 2 sides,
the back and below.

RETURN

483 mm x 483 mm 45T mm
19" 19" 17 34
451 mm

17 3/4"

Top View

559 mm
5"
25 mm
1" ‘T
SUPPLY 25 mm

508 mm x 508 mm
20" x 20"

q

559

mm
2"

KOE
A=35mm @44 mm
13/8"&1 3/4"

B=29 mm
7/8"





Models with Standard Motor (PSC)1
	Product #
	Volts (1 ph) 

2 wires
	kW (BTU/hr)
	Amp including

motor
	Motor 

HP
	Static Pressure at 0.5" W.C.
	Weight
	Qty

	
	
	
	
	
	Motor Speed2
	cfm3
	lb
	kg
	

	OFEA100001
	240/208
	10.0 (34120) / 7.5 (25628)
	44.0 / 38.2
	1/3
	Low
	595
	105
	48
	

	OFEA150001
	240/208
	15.0 (51180) / 11.3 (38442)
	64.7 / 56.1
	1/3
	Low
	595
	105
	48
	

	OFEA180001
	240/208
	18.0 (61420) / 13.5 (46130)
	78.4 / 68.0
	1/3
	Low
	595
	105
	48
	

	OFEA200001
	240/208
	20.0 (68240) / 15.0 (51256)
	85.4 / 74.1
	1/3
	Low
	595
	105
	48
	

	OFEA23000-1H1
	240/208
	23.0 (78480) / 17.3 (58944)
	102.4 / 88.7
	1
	Medium/Low
	1275
	105
	48
	

	OFEA25000-1H1
	240/208
	25.0 (85360) / 18.8 (64070)
	109.4 / 94.8
	1
	Medium/Low
	1275
	105
	48
	

	OFEA27000-1H1
	240/208
	27.0 (92130) / 20.3 (69195)
	119.5 / 103.6
	1
	Medium/Low
	1275
	105
	48
	

	OFEA30000-1H1
	240/208
	30.0 (102432) / 22.5 (76884)
	130.2 / 112.8
	1
	Medium/Low
	1275
	105
	48
	


Models with Variable Speed Motor (ECM)

	Product #
	Volts (1 ph) 

2 wires
	kW (BTU/hr)
	Amp including

motor
	Motor 

HP
	Motor Speed
	Weight
	Qty

	
	
	
	
	
	
	lb
	kg
	

	OFEA10000-VV
	240/208
	10.0 (34120) / 7.5 (25628)
	43.7 / 38.6
	1/2
	Variable
	105
	48
	

	OFEA15000-VV
	240/208
	15.0 (51180) / 11.3 (38442)
	64.4 / 56.5
	1/2
	Variable
	105
	48
	

	OFEA18000-VV
	240/208
	18.0 (61420) / 13.5 (46130)
	78.1 / 68.4
	1/2
	Variable
	105
	48
	

	OFEA20000-VV
	240/208
	20.0 (68240) / 15.0 (51256)
	85.1 / 74.5
	1/2
	Variable
	105
	48
	

	OFEA23000-1HVV
	240/208
	23.0 (78480) / 17.3 (58944)
	101.7 / 89.6
	1
	Variable
	105
	48
	

	OFEA25000-1HVV
	240/208
	25.0 (85360) / 18.8 (64070)
	108.7 / 95.7
	1
	Variable
	105
	48
	

	OFEA27000-1HVV4
	240/208
	27.0 (92130) / 20.3 (69195)
	118.8 / 104.5
	1
	Variable
	105
	48
	

	OFEA30000-1HVV4
	240/208
	30.0 (102432) / 22.5 (76884)
	129.5 / 113.7
	1
	Variable
	105
	48
	


1 Installation is forbidden outside Québec.

2 Factory settings.

3 Refer to the installation instructions and owner’s manual for complete air flow tables.

4 These models come with two supply terminal blocks. It is possible to do a field conversion from two to one supply (requires copper wire). Refer to the installation instructions and owner’s manual for details.

Standard color is grey.

Options
	Product  # 

En trousse 
	Description
	Qty

	OFE-SO2
	Downflow sub-base
	

	OFE-AK2
	Adaptor kit for 17 in. x17 in. duct
	

	OFE-TM
	Diagnostic tool for ECM motor
	


Wall Controls

Compatible with any low voltage thermostat.
Specifications may change without prior notice

	Project:
	Approval

Information

	Consultant Engineer:
	

	Electrician Contractor:
	

	Distributor:
	

	Date:
	

	Contact in Ouellet:
	

	Comments:


180, 3e Avenue, L’Islet (Québec) G0R 2C0 CANADA • Phone: 1 800 463-7043 • Fax: 1 800 662-7801 • info@ouellet.com • www.ouellet.com

