Series

ORF-P

120V Preassembled Constant Wattage Heating Cable for Pipes

Features

Nominal voltage	-	120V.

Linear density	-	7 Watts per foot.

Cold lead length	-	30 in. (0.76 m).

Outer jacket	-	PVC.

Bus wire	-	Nickel plated copper.

Minimum bend radius	-  5/16 in. (8 mm).

Included hardware	-	Built-in bi-metal thermostat energizes the cable when temperature falls 
below 4 °C (40 °F).

- Grounded 3-pronged plug with indicator light to show when the cable is on.

Installation	-	Never cut or shorten the heating cable.

- Not for residential use in the United States.

- Installation under the insulation of the pipe.

- For indoor and outdoor applications.

- Minimum installation temperature: 0 °C (34 °F).

Warranty	-	1-year basic warranty on the heating cable.

Application	- 	Metallic and non-metallic pipes.

- Helps to prevent damage caused by frozen pipes.

Models

| Product #1 | Length |  | Watts | Qty |
| --- | --- | --- | --- | --- |
|  | ft. | m |  |  |
| ORF-P003 | 3 | 0.9 | 21 |  |
| ORF-P006 | 6 | 1.8 | 42 |  |
| ORF-P009 | 9 | 2.7 | 63 |  |
| ORF-P012 | 12 | 3.7 | 84 |  |
| ORF-P015 | 15 | 4.6 | 105 |  |
| ORF-P018 | 18 | 5.5 | 126 |  |
| ORF-P024 | 24 | 7.3 | 168 |  |
| ORF-P030 | 30 | 9.0 | 210 |  |
| ORF-P040 | 40 | 12.2 | 280 |  |
| ORF-P060 | 60 | 18.3 | 420 |  |
| ORF-P080 | 80 | 24.4 | 560 |  |

1 Must be plugged into a 120V outlet fitted with ground fault protection device (GFCI).

Options

| Product # Kit | Description | Qty |
| --- | --- | --- |
| ELB-02B | Self-adhesive glass fiber tape, max. temp. = 90 °C (194 °F), 50 m (165 ft.) |  |
| ELB-06C | Self-adhesive aluminum tape, max. temp. = 80 °C (176 °F), 50 m (165 ft.) |  |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |