Series

ORF-R

120V Preassembled Constant Wattage Heating Cable

for Roof and Gutter De-icing

Features

Nominal voltage	-	120V.

Linear density	-	5 Watts per foot.

Cold lead length	-	30 in. (0.76 m).

Outer jacket	-	PVC.

Bus wire	-	Nickel plated copper.

Bending radius, minimum	-	1/2 in. (12 mm).

Included hardware	-	Roof clips for cable and spacers.

- Grounded 3-pronged plug with indicator light to show when the cable is on.

Installation	-	Never cut or shorten the heating cable.

- For residential outdoor applications only.

- Minimum installation temperature: 0 °C (32 °F).

Warranty	-	1-year basic warranty on the heating cable.

Application	-	Roof and gutter de-icing.

Models

| Product #1 | Length |  | Watts | Qty |
| --- | --- | --- | --- | --- |
|  | ft. | m |  |  |
| ORF-R020 | 20 | 6.1 | 100 |  |
| ORF-R030 | 30 | 9.1 | 150 |  |
| ORF-R060 | 60 | 18.3 | 300 |  |
| ORF-R080 | 80 | 24.4 | 400 |  |
| ORF-R100 | 100 | 30.5 | 500 |  |
| ORF-R120 | 120 | 36.6 | 600 |  |
| ORF-R140 | 140 | 42.7 | 700 |  |
| ORF-R160 | 160 | 48.8 | 800 |  |
| ORF-R180 | 180 | 54.9 | 900 |  |
| ORF-R200 | 200 | 61.0 | 1000 |  |
| ORF-R240 | 240 | 73.2 | 1200 |  |

1 Must be plugged into a 120V outlet fitted with ground fault protection device (GFCI).

Options

| Product # Kit | Description | Qty |
| --- | --- | --- |
| KIT-RF-CLIP | Roof clips (20) and spacers (15) for constant wattage heating cables |  |
| RCR-U | Roof and gutter sentry for automatic de-icing control with humidity probe |  |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |