Series

OSKHH150R

HRV - Heat Recovery Ventilator with Recirculation

Top Port Model

Features

Air flow and performance	-	Fully adjustable air ﬂow (152 cfm @ 0 in.w.g.).

- 84% sensible recovery effectiveness at 0 °C (32 °F) 30 L/s,

62% sensible recovery effectiveness at -25 °C (-13 °F) 33 L/s.

- Recirculation defrosts.

Voltage	-	120V, 60Hz, 1.5A, 172W.

Benefit	-	Reduce heating demand in winter and cooling demand in summer.

-  Whisper-quiet: factory calibrated and balanced.

-	Easy installation and maintenance.

-	Designed and manufactured in Canada to operate in variable climatic conditions.

Construction	-	Door and housing in corrosion-resistant galvanized steel,

24-gauge, pre-painted neutral white.

- 	Convenient, screwless hinged door.

-	Ultra-thin: 14 1/8 in. (359 mm).

- 	Easy installation: 59.5 lb (27 kg).

- 	Port size: 5 in. (127 mm).

- 	Top port model only.

- 	Merv8 washable filters.

Motor	-	Energy-efficient, sealed European motor with closed housing and automatic reset

thermal protection.

- Balancing without flow keys. 100% electronic adjustment. Pressure ports integrated

into the door for precise, easy balancing.

Heat recovery core	-	Polymer counterflow heat recovery core offers high energy efficiency for superior performance.

-  Washable in soapy water to recover its full effectiveness without deterioration

or contamination.

Control/timer	-	2 control choices available:

-  Optional Decora-style wall-mounted control/timer (SA-LT15).

INTERMITENT (20-40) – CONTINUOUS – OFF – Timer (20-40-60 minutes).

-  Optional digital Decora-style wall-mounted control/timer (SA-LCD15).

INTERMITENT (20-40) – CONTINUOUS – Humidity – Recirculation – OFF.

Installation	-		Can be wall-mounted or suspended.

- Set of nylon suspension straps included for installation.

- Fits into a 24 in. (610 mm) deep wardrobe.

- Removable terminal block for connection to a furnace, supplied with the device.

Setting	-	Balance the unit using the supplied instruction.

- All the installer needs are the required flow rates, a manometer,

and 5 minutes to adjust and balance the system.

- Simply specify the air flow rates for LOW SPEED and HIGH SPEED.

- The static pressure of the system should ideally be less than or equal to the static

pressure indicated by the graph for HIGH SPEED.

- You don't have to worry about static pressure to limit air flow or adding flow keys as

other devices. All you must do is provide ducting that doesn't limit maximum flow,

and the unit's adjustments will allow you to fine-tune flow rates.

Warranty	-	Motor 5-year. Other parts except filters and pre-filters 1-year.

Application 	-	The ideal solution for high-rise apartments and small to medium-size homes.

-  The most efficient heat recovery ventilator on the market. Ideal for projects

requiring very high energy performance to benefit from all available credits and

subsidies.

- Purifies the air by expelling many pollutants, dust, and allergens to the outside,

while ensuring an equivalent supply of fresh air in the home.

*Important note: 	- No controls are included with the purchase of an air exchanger.

SA-LT15 and SA-LCD15 controls must be purchased separately.

OSKHH150R

Top Port Model

HRV OSKHH150R - Curves at highest speed

Energy ratings - HVI certified

| Supply temperature |  | Air flow |  | Power | Sensible recovery (SRE)1 | Adjusted sensible recovery |
| --- | --- | --- | --- | --- | --- | --- |
| °C | °F | cfm | L/s | W | % | % |
| Heating |  |  |  |  |  |  |
| 0 | 32 | 64 | 30 | 70 | 84 | 93 |
| 0 | 32 | 96 | 45 | 98 | 81 | 88 |
| 0 | 32 | 134 | 63 | 134 | 79 | 86 |
| -25 | -13 | 70 | 33 | 105 | 62 | 66 |

Model

| Product # | cfm | Recirculation | SRE1 0 °C (32 °F) 30 L/s | SRE1 -25 °C (-13 °F) 33 L/s | Port |  | Volts | Hertz | Amp. | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | in. | mm |  |  |  | lb | kg |
| HRV Heat recovery ventilator 150 cfm with recirculation and top port |  |  |  |  |  |  |  |  |  |  |  |
| OSKHH150RT | 152 | Yes | 84% | 62% | 5 | 127 | 120 | 60 | 1.5 | 59.5 | 27 |

1 SRE: Sensitive Recovery Effectiveness.

Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| KIT-OSA-INS-5 | - | Basic installation kit for air exchanger, 5 in. ducts |  |
| KIT-SA-FI-150 | - | Kit of 2 Merv8 filters for 150R air exchanger |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

Controls sold separately

| Product # | Description | Qty |
| --- | --- | --- |
| SA-LT152 | Decora-style wall-mounted control/timer for residential air exchangers |  |
| SA-LCD152 | Digital Decora-style wall-mounted control/timer for residential air exchangers |  |

2 Must be purchased separately

### Others

| Type | Quantity | Model and description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrical Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact at manufacturer: |  |
| Comments: |  |