[image: image4.png]AR CerTIFIED®




[image: image5.png]N\
INVERTER
N



[image: image6.png]





Approval Drawings

Series
[image: image7.jpg]@gsCOLD

s CLIMATE




OTP-F-R32
Flexx R32 Central Heat Pump
[image: image8.png]




          Indoor Unit         
         Outdoor Units
[image: image9.png]




  Air Handler
[image: image10.png]Includes

JEADHERNEBNE

Self-  Comprehensive High ESP Wide High Intelligent Energy Flexible A2L
Diagnosis Protection Operation Efficiency ~ Defrosting Saving Connecting Pipe  safety
Range Function Available sensor





Features
Operation mode
- 
Cooling, heating, dehumidifying, fan and auto. 
Nominal capacity
-
24000, 36000, 42000, 60000 BTU/hr.
Voltage
-
230/208V, 60 Hz, 1-phase.
Indoor unit
Air Handler


-
Compact and space-saving design.


-
Compatible with most standard 24V thermostats on the market.

-  A2L leak detection sensor, included
Outdoor unit
-
Variable speed rotary compressor (INVERTER), with overload protection.


-
Electronic expansion valve for precise control of refrigerant flow.


-
Condenser fins covered with a hydrophilic coating for improved corrosion resistance.


-
Large diameter quiet axial-flow fan for more efficient heat exchange.

-
Smart pan heater to prevent ice formation.


-
Compressor blanket and preheating technology.

-
Installation on aluminum wall bracket or ground stand (optional).
Connection piping
-
Minimum piping length of 24.6 ft. (7.5 m).

· Flared connections.

Control 
-
Operating temperature: 


-
 Cooling: -15 to 54 °C (5 to 129 °F).


- Heating: -30 to 24°C (-22 to 75 °F).

-
Temperature display format: °C or °F.

Warranty
-
10-year basic warranty on parts and 10-year basic warranty on the compressor (labour not included).
· Labour warranty (sold separately) from the 1st to 10th year, on parts and labour with condenser replacement if the compressor fails.
Application
-
Private residence, condo, commercial building, office building, restaurant.
[image: image11.png]



Models - Outdoor Unit 
	Product #
	OFLX1-H36A-O
	OFLX1-H60A-O

	Qty
	
	

	Technical Performance 

	Refrigerant type                                                                                                R32                                                                                                   R32

	Cooling capacity
	Nominal
	BTU/hr
	24000
	34000
	48000
	54000

	
	Min. – max.
	W (BTU/hr)
	3.5 – 8.8
(12000 - 30000)
	3.2 - 10.8

(18000 - 37000)
	9.96 - 14.65

(34000 - 50000)
	10.26 – 16.12

(35000 - 55000)

	Heating capacity
	Nominal
	BTU/hr
	24000
	34000
	48000
	54000

	
	Min. – max.
	W (BTU/hr)
	3.5 – 8.8
(12000 - 30000)
	5.2 – 11.1
(18000 - 38000)
	9.86 – 15.24

(34000 - 52000)
	10.26 - 17.58

(35000 - 60000)

	Power supply
	V / Ph / Hz
	230/208 / 1 / 60
	230/208 / 1 / 60
	230/208 / 1 / 60
	230/208 / 1 / 60

	SEER2
	-
	18
	18
	19
	18.5

	HSPF2
	-
	10
	10
	10
	10.5

	EER2
	-
	12.5
	12
	12
	11.7

	COP
	-
	3.6
	3.2
	3.4
	3.4

	MCA
	A
	30
	30
	45
	45

	Max. over-current protection (MOCP)
	A
	27.7
	27.7
	39.9
	39.9

	AHRI number
	-
	216626170
	216440572
	216626171
	216440574

	ENERGY STAR
	-
	(
	(
	(
	(

	Refrigerant volume (R32)
	lb (oz)
	4.4 (102.3)
	4.4 (102.3)
	8.82 (162.3)
	8.82 (162.3)

	Connection pipes
	Liquid
	in.
	3/8
	3/8
	3/8
	3/8

	
	Gas
	in.
	3/4
	3/4
	3/4
	3/4

	Net weight
	lb (kg)
	187.5 (85)
	187.5 (85)
	241.4 (109.5)
	241 (109.5)

	Sound level
	dB (A)
	60
	60
	62
	63

	Compressor power input
	W
	14
	19.7
	30.1
	30.1

	Compressor LRA
	A
	16
	16
	19
	19

	Compressor RLA
	
	19.7
	19.7
	30.1
	30.1

	Fan motor power output
	A
	1.5
	1.5
	1.5 x 2
	1.5 x 2

	Cooling nominal power input
	W
	14.6
	22
	30
	31

	Heating nominal power input
	W
	19.2
	25.6
	32
	32

	Operating ambient temperature in cooling
	°C (°F) 
	-15 to 54 (5 to 129)

	Operating ambient temperature in heating
	°C (°F)
	-30 to 24 (-22 to 75)

	Maximum pipe length between outdoor and indoor units
	ft. (m)
	99 (30)
	99 (30)
	99 (30)
	99 (30)

	Maximum height between outdoor 
and indoor units
	ft. (m)
	49 (15)
	49 (15)
	49 (15)
	49 (15)


Models - Indoor Unit (Air handler)

	Product # 
	OAHX1-H24A-I
	OAHX1-H36A-I
	OAHX1-H48A-I
	OAHX1-H60A-I

	Qty
	
	
	
	

	Electrical

data
	Power supply
	V / Ph / Hz
	230/208 / 1 / 60
	230/208 / 1 / 60
	230/208 / 1 / 60
	230/208 / 1 / 60

	
	Nominal curent
	Cooling
	W
	1
	1.6
	2.1
	2.8

	
	
	Heating
	A
	1
	1.5
	1.9
	2.5

	
	Max. over-current protection (MOCP)
	A
	15
	15
	15
	15

	
	Voltage min./max.
	V
	187 / 253
	187 / 253
	187 / 253
	187 / 253

	Sound level
	dB (A)
	50
	51
	52
	53

	Refrigerant
	Type
	-
	R32

	
	Control
	-
	Thermal expansion valve

	ENERGY STAR
	-
	(
	(
	(
	(

	Air flow
	cfm
	760
	1000
	1200
	2550

	
	m3/hr
	1291
	1700
	2040
	2550

	HPS (High Static Pressure)
	Range
	Pa
	0 to 250
	0 to 250
	0 to 250
	0 to 250

	
	
	inH2O
	0 to 1
	0 to 1
	0 to 1
	0 to 1

	Fan motor
	Model
	-
	B-SWZ150E
	B-SWZ150E
	B-SWZ120E
	B-SWZ120E

	
	Transmission type
	-
	Direct transmission

	
	Speed
	RPM
	800
	800
	800
	800

	
	Electric power
	W
	124
	124
	2 x 124
	2 x 124

	
	Full load current (FLA)
	A
	3
	3
	2 x 1.1
	2 x 1.1

	Evaporator
	Material
	-
	Copper tube with inner groove – Aluminum fins

	
	Surface area
	sq. ft. (m²)
	9.94 (0.92)
	9.94 (0.92)
	11.41 (1.06)
	11.41 (1.06)

	
	Pipe diameter
	in.
	3/8
	3/8
	3/8
	3/8

	
	Fins type
	-
	Hydrophilic

	
	Dimension (L x H x l)
	in.
	30 3/32×36 13/32×1 1/2
	30 3/32×36 13/32×1 1/2
	39 1/2×36 13/32×2 19/64
	39 1/2×36 13/32×2 19/64

	System operation control
	-
	 Wired controller

	Weight
	Net 
	lb (kg)
	134.5 (61.5)
	163 (74)
	200.5 (91)
	200.5 (91)

	
	Gross
	lb (kg)
	143 (65.5)
	178.5 (81)
	218 (99)
	218 (99)


Options
	Product # 

Kit
	Description
	Qty

	21-4227-01
	5kW electric auxiliary heat module
	

	21-4216-03
	10kW electric auxiliary heat module
	

	21-4217-01
	15kW electric auxiliary heat module
	

	21-4228-01
	20kW electric auxiliary heat module
	

	OTP-FKCP
	Adaptor for welding option on suction line
	

	OTP-B04537
	Terminal block kit
	

	OTP-R02P033
	24V thermostat
	


Compatibility table for electrical heating elements
	Electric heater power
	OAHX1-H24A-I
	OAHX1-H36A-I
	OAHX1-H48A-I
	OAHX1-H60A-I

	5kW
	X
	X
	X
	X

	10kW
	
	X
	X
	X

	15kW
	
	
	X
	X

	20kW
	
	
	X
	X


[image: image1.png]Outdoor Unit

OFLX1-H36A-0 OFLX1-H60A-0
=
=
=
=
c =]
£5 >
= f i
% ———— E
= (S
S |
= o
= Top Vi
Front View Side View =] op View
=
o =
.
Y
G . E Front View Side View
3

Top View




	Outdoor Unit
	A
	B
	C
	D
	E
	G

	
	in.
	mm
	in.
	mm
	in.
	mm
	in.
	mm
	in.
	mm
	in.
	mm

	OFLX1-H36A-O
	38 31/32
	990
	16 13/16
	427
	37 51/64
	960
	29 49/64
	756
	15 35/64
	395
	14 9/16
	370

	OFLX1-H60A-O
	35 7/16
	900
	16 17/64
	413
	49 39/64
	1260
	22 7/16
	570
	14 7/8
	378
	13 25/64
	340


[image: image2.png]Indoor Unit - Air Handler





	Indoor Unit
Air Handler
	Dimensions

	
	W
	D
	H
	A
	B

	
	in.
	mm
	in.
	mm
	in.
	mm
	in.
	mm
	in.
	mm

	OAHX1-H24A-I
	18 7/64
	460
	21 1/4 
	540
	43 1/2
	1105
	11 5/8 
	295
	16 47/64
	425

	OAHX1-H36A-I
	21 ¼
	540
	21 1/4
	540
	48 3/16
	1224
	11 5/8
	295
	20
	508

	OAHX1-H48A-I
	24 3/4 
	630
	21 1/4 
	540
	52
	1321
	11 5/8 
	295
	20 
	508

	OAHX1-H60A-I
	
	
	
	
	
	
	
	
	
	


Specifications may change without prior notice

	Project:
	Approval

Information

	Consultant Engineer:
	

	Electrical Contractor:
	

	Distributor:
	

	Date:
	

	Contact at manufacturer:
	

	Comments:


[image: image3.jpg]



-15˚C�5˚F





Performance up to 





Up to





-30˚C�-22˚F





19 SEER2





10.5 HSPF2





-15˚C�5˚F








[image: image12.png]


180, 3e Avenue, L’Islet (Québec) G0R 2C0 Canada • Phone: 1 800 463-7043 • Fax: 1 800 662-7801 • info@ouellet.com • www.ouellet.com

[image: image13.png]e

NCOOLING &

Cooling Heating _—




[image: image14.jpg]@DUELLET



