[image: image1.png]


[image: image3.jpg]@DUELLET



[image: image3.jpg]

Approval Drawings

Series

OVOS
Stylish Wall Fan Heater
[image: image4.jpg]



[image: image5.emf]









Features

Color
-
Standard: white.
Finish
-
Standard: epoxy/polyester powder paint.
Voltage
-
120V, 208V, 240/208V, 1-phase.

Construction
-
22-gauge steel cabinet, 20-gauge steel grille.


-
High-limit temperature control with manual reset.


- 
Additional one-time thermal fuse.

-
Highly secure façade.
Fan
-
Quiet fan.


Heating element
-
Nickel-chromium heating element provides instant heat.

Control
-
Built-in electronic thermostat (-TH models).
- 
Wall thermostat (not included, the use of an electronic thermostat is strongly recommended).


 
Note: If an electronic wall-mounted thermostat is used, it must be compatible with forced-air heaters.
Installation
-
6 in. (150 mm) minimum clearance to ﬂoor or any adjacent surfaces.

-
May be recessed or surface-mounted with optional surface mounting box.

- Back box (wall can) is available for installation during wall construction and the 



Heater Assembly Only and the grille can be installed after walls are finish.

- Quick and easy installation easily retrofits most popular models.

- 
Wattage can be selected onsite for models without control only.
Warranty
-
5-year warranty, (3-year on electronic components).
Application
-
Kitchen, family room, bathroom (model with timer and thermostat), entrance way, office.
[image: image2.png]100 mm

230 mm 35 mm 315/16"
91/16" 13/8"
T ™ i
—
&
= -
=
=2
=
=
=1
=
=0 —
317 mm =
12172" = —
|-
=
|-
. <
79 mm
‘ 31/8"
] @ J ]
152 mm 25 mm 21 mm
6" MIN I B ™~ 13/16" 54 mm 106mm |

j— 21/8" 43/16"

Front View Side View Bottom View




Models 

	 Product #
Without 

thermostat2
	Qty
	Watts1
	BTU/hr
	Product #

Built-in electronic

 thermostat
	Qty
	Watts
	Volts
	BTU/hr
	Weight

	
	
	
	
	
	
	
	
	
	lb
	kg

	OVOSU1002BL
	
	1000 or 500
	3413 or 1706
	OVOSU1002BL-TH
	
	1000
	120
	3413
	7.2
	3.3

	OVOSU1502BL
	
	1500 or 750
	5119 or 2560
	OVOSU1502BL-TH
	
	1500
	120
	5119
	7.2
	3.3

	OVOSU1008BL
	
	1000 or 500
	3413 or 1706
	OVOSU1008BL-TH
	
	1000
	208
	3413
	7.2
	3.3

	OVOSU1508BL
	
	1500 or 750
	5119 or 2560
	OVOSU1508BL-TH
	
	1500
	208
	5119
	7.2
	3.3

	OVOSU1000BL
	
	1000/750 or 500/375
	3413/2560 or 1706/1280
	OVOSU1000BL-TH
	
	1000/750
	240/208
	3413/2560
	7.2
	3.3

	OVOSU1500BL
	
	1500/1125 or 750/560
	5119/3839 or 2560/1911
	OVOSU1500BL-TH
	
	1500/1125
	240/208
	5119/ 3839
	7.2
	3.3

	Heater Assembly Only3 – Must be installed with back box (wall can) and grille available in Options 

	OVOSU1002-HAO
	
	1000 or 500
	3413 or 1706
	OVOSU1002-TH-HAO
	
	1000
	120
	3413
	4.2
	1.9

	OVOSU1502-HAO
	
	1500 or 750
	5119 or 2560
	OVOSU1502-TH-HAO
	
	1500
	120
	5119
	4.2
	1.9

	OVOSU1000-HAO
	
	1000/750 or 500/375
	3413/2560 or 1706/1280
	OVOSU1000-TH-HAO
	
	1000/750
	240/208
	3413/2560
	4.2
	1.9

	OVOSU1500-HAO
	
	1500/1125 or 750/560
	5119/3839 or 2560/1911
	OVOSU1500-TH-HAO
	
	1500/1125
	240/208
	5119/ 3839
	4.2
	1.9


Standard color is white.

Heater assembly must be installed with compatible back box (KIT-OVO-BB (-10)) and grill (KIT-OVOS(1X0X-TH)-GRIXX)

1 Wattage can be selected onsite for models without control only.
2Requires non pulsing wall-mounted thermostat. Please refer to the Wall Thermostats Reference Chart at the end of the catalog.
3 Heater Assembly Only (-HAO) available for United States market only.

Options
	Product # 
Kit
	Description
	Qty

	KIT-OVO-BB
	Back box (wall can)
	

	KIT-OVO-BB-10
	Back box (wall can) 10-pack
	

	White (BL)
	
	

	KIT-OVO-AP1BL
	Wall adapter plate 18 1/2'' x 22'' white
	

	KIT-OVO-AP3BL
	Wall adapter plate 12'' x 21 1/4'' white
	

	KIT-OVO-BSBL
	Surface mounting box for OVO and OVOS white
	

	KIT-OVOS-GRIBL
	Grille OVOS, white
	

	KIT-OVOS1000-TH-GRIBL
	Grille OVOS-TH 1000/750W 240/208V white
	

	KIT-OVOS1002-TH-GRIBL
	Grille OVOS-TH 1000W 120V white
	

	KIT-OVOS1500-TH-GRIBL
	Grille OVOS-TH 1500/1125W 240/208V white
	

	KIT-OVOS1502-TH-GRIBL
	Grille OVOS-TH 1500W 120V white
	


Others

	Type
	Quantity
	Model et description

Complementary informations
	Volts
	Phase
	Watts
	Length

mm/in.
	Informations

Color/installation

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	


Specifications may change without prior notice

	Project:
	Approval

Information

	Consultant Engineer:
	

	Electrician Contractor:
	

	Distributor:
	

	Date:
	

	Contact in Ouellet:
	

	Comments:


180, 3e Avenue, L’Islet (Québec) G0R 2C0 CANADA • Phone: 1 800 463-7043 • Fax: 1 800 662-7801 • info@ouellet.com • www.ouellet.com

