- Series

OTH3600-GA-GT

Wireless communication programmable electronic thermostat

for floor heating system

Neviweb® is a registered trademark

of Sinopé Technologies Inc.

in Canada and the United States.

Compliance with standard

CAN/CSA-C828-13

- Features

Type	-	Wireless communication programmable, electronic, double-pole, 4-wire thermostat

for ﬂoor heating system.

Color	-	Standard: white.

Voltage	-	120/208/240V, 50/60Hz.

Rating	-	15A at 120/208/240V.

Control	-	15-minute cycle.

-		Resolution and precision: ± 0.5 °C (1 °F).

-	Auxiliary contact offers the possibility of controlling supplemental heat sources to be added by either using a slave module (TR1310-120/240GA) for heated floors or using a low voltage relay and a low voltage transformer for an electric heater.

Temperature control mode	-	A mode: Ambient temperature control and possibility of limiting the temperature of the floor using an external temperature sensor.

- F mode: Floor temperature control using and external temperature sensor and

possibility of limiting the ambient temperature.

Temperature range	-	Floor: 5 °C (40 °F) to 30 °C (86 °F).

- Ambient: 5 °C (40 °F) to 30 °C (86 °F).

Display	-	Digital display of actual and setpoint temperature (°C or °F), heating level, actual 			mode and clock (12 or 24-hour format).

- Temperature setpoint and settings are permanently recorded.

- GFCI fault indicator.

- Backlit.

Program	-	Programming features are accessible only through the neviweb® Web platform.

Connection requires a GT125 interface.

Installation	-	4-wire thermostat with ambient and floor sensor.

Warranty	- 	5-year warranty against defects.

Models

| Product # | Qty | Description | Amp. | Volts | Protection |
| --- | --- | --- | --- | --- | --- |
| OTH3600-GA-GT2 |  | Wireless communication programmable electronic thermostat for floor heating system | 15 | 120/208/240 | Class A GFCI 5 mA1 |
| TR1310-120/240GA |  | Slave unit for floor heating system | - | - | - |
| GT125 |  | Web interface that provides remote access to all your neviweb®–compatible products | - | - | - |

1 GFCI: Ground fault circuit interrupter.

2 15' (4.6 m) floor sensor included.

Standard color is white.

- Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |